Experiment No. 1 – Flashing of LED using AT89C51

Objectives:

- To study the basic operation of the AT89C51 microcontroller.

- To interface an LED with the microcontroller.

- To generate LED blinking using a software delay routine.

PROGRAM:

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT: