Experiment No. 2 – Generation of Square Wave using Proteus

- To generate a square wave using the AT89C51.

- To study port toggling operation.

- To observe the waveform using an oscilloscope in Proteus.

Program:

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

Output: